City densification and temporal dynamics of traditional inner core of Ibadan, Nigeria

Bamiji Michael Adeleye1, Ayobami Abayomi Popoola2, Zitta Nanpon3, Kolawole Adebayo Shittu4, Funke Jiyah5, Hangwelani Magidimisha-Chipungu2

1Department of Architecture and Physical Planning Makerere University Kampala

2SARCHI Chair for Inclusive Cities, University of KwaZulu-Natal Durban South Africa

3Department of Survey and Geo-Informatics, Federal University of Technology Minna, Nigeria

4Department of Environmental Health Science, Kwara State University, Nigeria

5Department of Urban and Regional Planning, University of Ibadan, Nigeria

Correspondence: Ayobami Abayomi Popoola (email: bcoolay2@yahoo.com)

Received: 18 August 2022; Accepted: 8 May 2023; Published: 31 May 2023

Abstract

The conversion of vegetation land cover contributes to the retention of solar radiation in the environment, resulting in the formation of the urban heat island. This study analyses the distribution pattern of urban heat island in the traditional core of Ibadan amidst the rapid urbanization experienced. The remote sensing tool was used to analyse the trend of land surface temperature, normalized difference built-up index, and normalized difference vegetation index for the traditional Ibadan's core between the year 2000 and the year 2020. This reveals that increasing built-up areas will continue to strengthen the effects of urban heat island in the traditional core of Ibadan, while vegetated land covers will weaken the effects of urban heat island. This is because anthropogenic activities resulting from rapid urbanisation has adversely altered the natural landscape in the traditional core of Ibadan. This alteration manifests in converting vegetation land covers into physical developments and other impervious surfaces by the increasing urban population. Thus, increasing the land surface temperature. The maximum average land surface temperature of 35.34℃, 36.62℃ and 31.86℃ were record for the years 2000, 2013 and 2020 respectively. This study further recommends that urban greening and proper urban planning should be encouraged in the traditional core of Ibadan.

Keywords: Land Surface Temperature, Normalized Difference Built-up Index, Normalized Difference Vegetation Index, traditional core, urban heat island

Introduction

Cities in the world are experiencing rapid urbanization. It is envisaged that by the year 2030, the world’s urban population would have increased to 5 billion and 6.4 billion by 2050 (UN-Habitat, 2009). To meet the demand of the increasing urban population, urban infrastructures and physical developments need to be put in place (Senanayake et al., 2013). Rapid urbanization is often accompanied by adverse environmental impacts, ranging from environmental pollution, replacement of natural landscape with various built-up structures, breakdown of ecological cycles, and climate change (Senanayake et al., 2013; Fang et al., 2021; Chang et al., 2023). The latter (climate change) environmental impacts of rapid urbanization have always been a major source of concern to urban planners, governments, and city stakeholders (Anibaba et al., 2019).

This is because urbanization is a significant propellant of climate change and the conversion of the natural landscape into other land uses (Fashae et al., 2020; Kaplan et al., 2018; Senanayake et al., 2013). The removal of land cover as a result of increase anthropogenic activities which are brought about by rapid urbanization is a significant contributor to urban heat island (Fabeku et al., 2018; Folorunsho et al., 2017; Kaplan et al., 2018; Senanayake et al., 2013; Popoola, 2019). In Ibadan, Popoola (2019) argued that anthropogenic activities such as indiscriminate bush burning and urbanisation resulting to green space ‘grabbing, invasion and conversion’ contribute to climate anomalies. Urbanization, both in the human population and geographical extent, often transforms the land covers from its natural state into various impervious surfaces (Kafi et al., 2014). These impervious surfaces increase the retention of solar radiation, decrease in evapotranspiration, increase runoff, release anthropogenic heat, and heat conductivity, which result in urban heat island (Weng et al., 2004; Zhang et al., 2011; Olorunfemi et al., 2020). Reporting on evapotranspiration in South Western Nigeria city of Ekiti and Ogbomoso and climate change, urbanisation have been attributed to anomalies in natural functioning of ecosystem (Ogunbode & Ifabiyi, 2019; Nwosu & Oshunsanya, 2020; Olorunfemi, 2020)

This phenomenon is not farfetched in the core of Ibadan, Oyo State. The city is characterized by high residential density and intensive commercial activities. Over the years, the core of Ibadan, Nigeria, has experienced unprecedented urbanization, which has resulted in the conversion of different land covers into various impervious surfaces. Thus, contributing to the city core thermal stress due to poor residential planning characterised by little or no airspace for ventilation (Fashae et al., 2020). The unplanned state of the core of Ibadan has intensified the relative warmth of the core as compared to the peri-urban areas (Folorunsho et al., 2017). The relatively high surface temperature in the urban centre compared to the rural surrounding is referred to as urban heat island (Anibaba et al., 2019; Schwarz et al., 2011; Yamamoto, 2006).

Chang et al. (2023) mentioned that despite importance of vegetation and environmental management to city stakeholder, its relationship with urbanisation and city planning remains poorly understood. In Ibadan, Nigeria, studies on urban heat island have focused on the metropolitan and its sub-urban area (Abegunde & Adedeji, 2015; Adetoro & Salami, 2018; Anibaba et al., 2019; Fashae et al., 2020). With emphasizes on the relationship between the land uses, land cover and surface temperature. This gap was filled in this study using normalised difference built-up area index which was effectively used as a quick alternative for mapping built-up areas in the core of Ibadan. This study aim is to analyse the distribution pattern of the land surface temperature of traditional core of Ibadan between 2000 and 2020. The specific objectives were: a) to spatially analyse urban heat island formation in the core of Ibadan between 2000 and 2020; b) to determine the normalized difference vegetation index for the traditional core of Ibadan between 2000 and 2020; and c) to determine the relationship between LST, NDBI, and NDVI for traditional core of Ibadan between 2000 and 2020.

Study area

The traditional core of Ibadan is referred to as the largest indigenous city in sub-Saharan Africa (Onibokun & Kumuyi, 1999). The traditional core of Ibadan is located within latitudes 7º 20ʹN and 7º 30ʹN of the equator and longitudes 3º 45ʹE and 4º 00ʹ E of the Greenwich meridian (See figure 1). The population of the traditional core of Ibadan was reported to be 1,338,659 by the National Population Commission in 2006 with the growth rate of 0.5 percent (NPC, 2006). The traditional core of Ibadan, which may also be referred to as the urban core of Ibadan, consists of five local government areas (Ibadan North, Ibadan Northeast, Ibadan Northwest, Ibadan Southeast, and Ibadan Southwest) with a land area of 135.95 km2. Since 2006, the traditional core of Ibadan has had a significant population growth and it was envisaged that the growth of the traditional core will increase from 1.3 m to 5.03 m by the year 2025 (Adelekan, 2016).

Source: Department of Urban and Regional Planning Federal University of Technology, Minna, 2019

Figure 1. Ibadan core and its Peri-Urban area

The rapid urbanization experienced within the traditional core of Ibadan is associated with socio-economic problems which increases the various planning problems in the core (Adelekan, 2016). The traditional core of Ibadan is predominantly occupied by the indigenes (Wahab & Popoola, 2019; Adelekan, 2016; Fourchard, 2003). The traditional core of Ibadan can be described as informal (organic) settlement which grew by the process of densification (Adelekan, 2016). The traditional core of Ibadan is densely populated and characterized by the presence of slum, poor landscape elements, poor housing, non-existence of drainages and sewages (Adelekan, 2016; Fourchard, 2003). According to Arimah (1994), 70-80 percent of the households in the traditional core of Ibadan reside in slums. Natural spaces in the houses in the traditional core of Ibadan are often provided for future expansion for more dwelling housing as family units grew larger (Adelekan, 2016). Some of the prominent political wards in the traditional cord of Ibadan are: Bere, Oje, Bode, Oke-Ado, Olorunsogo, Orita Merin, Ide Arere, Esu Awole, Oja Oba, Oke-Irefin, Eleka, Agban Gban (see Figure 2).

Source: Fourchard, 2003

Figure 2. The traditional core of Ibadan

Methodology

Data collection and post processing

The data sets used for this study are the Landsat Enhance Thematic Mapper for the year 2000 and the Landsat Operational Land Imager for the year 2013 and the year 2020. These data sets were downloaded from the United States Geological Survey website on path 191 and row 055 (See table 1). The data sets downloaded from the United States Geological Survey website were all cloud-free. On each of these data sets, the area of interest (the traditional core of Ibadan) was clipped out (Figure 3). On the clipped imageries of the study area, the normalized difference built-up area index (NDBI), average land surface temperature, and the normalized difference vegetation index (NDVI) were deduced for the three epochs (2000, 2013 and 2020) understudy.

Table 1. Properties of data sets

| Sensor | Year | Path and row | Resolution | Date acquired |
| --- | --- | --- | --- | --- |
| Enhance Thematic Mapper (ETM) | 2000 | P191/R055 | 30m | 06/02/2000 |
| Operational Land Imager (OLI) | 2013 | P191/R055 | 30m | 05/03/2013 |
| Operational Land Imager (OLI) | 2020 | P191/R055 | 30m | 05/04/2020 |

Source: United States Geological Survey, 2020

To calculate the normalized difference built-up area index, the short-wave infrared band and the near-infrared band (bands 6 and 5 for Landsat 8 and bands 5 and 4 for Landsat 7) were used. The mathematical expression for the computation of normalized difference built-up area index was:

NDBI=SWIR-NIR (Near Infrared)SWIR+NIR (Near Infrared)

The thermal band (band 6) was used to estimate the average land surface for the year 2000, while the thermal bands 10 and 11 at different wavelengths were used to estimate the average land surface temperature for the year 2013 and the year 2020.  At the first stage, the digital numbers were converted to spectral radiance using the equation Lλ = MLQcal + AL where Lλis top of atmosphere spectral radiance (Watts/(m2 * srad * µm)), MLis band-specific multiplicative rescaling factor from the metadata, Qcal is quantized, and calibrated standard product pixel values (DN), and AL is band-specific additive factors from the metadata. At the second stage, spectral radiance of the clipped imageries of the year 2000, 2013, and 2020 was converted to “At-Satellite brightness temperature” in Kelvin (TB) by using the inverse of the Planck function (NASA, 2012; Weng et al., 2004) which is expressed as:

TB=K2lnK_1Lλ+1

Where: TBis “At-Satellite brightness temperature”, Lλis top of atmosphere spectral radiance (Watts/(m2 * srad * µm)), K2,and K1 are band-specific thermal conversion constant from the metadata (See table 2). At the third stage, the land surface emissivity of the core of Ibadan was computed using the mathematical model expressed as E = 0.004 (Proportion of Vegetation) + 0.986, where PV is (NDVI - NDVImin /NDVImax - NDVImin)2.

Table 2. Metadata for Landsat 7 and Landsat 8

| Year | Landsat 7 | Band 6 | Band 6 |
| --- | --- | --- | --- |
| 2000 | Radiance Multiplier (M) | 0.067087 | 0.037205 |
|  | Radiance Add (A) | -0.06709 | 3.16280 |
|  | K1 | 666.09 | 1282.71 |
|  | K2 | 666.09 | 1282.71 |
|  | Landsat 8 | Band 10 | Band 11 |
| 2013 and 2020 | Radiance Multiplier (M) | 0.0003342 | 0.0003342 |
|  | Radiance Add (A) | 0.1 | 0.1 |
|  | K1 | 774.89 | 480.89 |
|  | K2 | 1321.08 | 1201.14 |

Source:  United States Geological Survey, 2000, 2013 and 2020

At the fourth stage, the land surface temperature of the core of Ibadan was computed using the signal window algorithm, which is expressed as LST=BT/1 + W* (BT/P) * ln (e) where BT is at Satellite temperature, W is the wavelength of emitted radiance (11.5µ), and P is h*c/s (1.438* 10˄- 2 m k)/14380.

Source: United States’ Geological Survey, 2020

Figure 3. Clipped imageries of the core of Ibadan

The NDVI for this study was calculated by finding the ratio between the red (R) or the visible spectrum and the near infrared (NIR) values. The equation used for the calculation of the NDVI is as:

NDVI=NIR Near Infrared-REDNIR Near Infrared+RED

Where NIR is the near infrared and Red is the visible spectrum. Bands 4 (Near infrared) and 3 (Red/visible spectrum) were used to compute the NDVI of the core of Ibadan in the year 2000. For the epochs 2013 and 2020, bands 5 (Near infrared) and 4 (Red/visible spectrum) were used to calculate the NDVI.

Result

Normalized Difference Built-up Index

The statistical results of the normalized difference built-up index of the core of Ibadan between the year 2000 and the year 2020 are presented in Table 3. Areas with high values NDBI signifies a large concentration of built-up areas in the core of Ibadan (Figure 4). The difference in NDBI in the three epochs were 0.4276, 0.6959 and 0.4349, respectively. The difference in NDBI indicates varying urban growth in the traditional core of Ibadan. The difference in NDBI also indicates that between the year 2000 and the year 2013, built-up areas in the core increased significantly.

Table 3. Statistical results of NDBI for the core of Ibadan between 2000 and 2020

| Year | Minimum | Maximum | Mean | Standard deviation |
| --- | --- | --- | --- | --- |
| 2000 | 0.0450 | 0.4726 | 0.2716 | 0.0382 |
| 2013 | -0.3080 | 0.3879 | 0.0001 | 0.0687 |
| 2020 | -0.2267 | 0.2082 | 0.0289 | 0.0355 |

Source: Authors’ Analysis, 2023

Source: Authors’ Analysis, 2023

Figure 4. NDBI for the traditional core of Ibadan between 2000 and 2020

Land Surface Temperature of the traditional core of Ibadan between 2000 and 2020

The study revealed that high land surface temperature was recorded in the innermost core of Ibadan compare to the relatively low average land surface temperature recorded in the periphery of the study area (Figure 4). The high average land surface temperature recorded in the innermost core was attributed to the compact urban form, population density, high built-up density and anthropogenic activities inherent in the innermost core of Ibadan. This assertion was affirmed by empirical studies that the land surface temperature in the heart of the city is higher than that of the edge or the periphery (Weng, 2004; Nuruzzaman, 2015; Zahabi & Pradhan, 2017; Folorunsho et al., 2017; Anibaba et al., 2019).

The study revealed that the average land surface temperature of the traditional core of Ibadan in the year 2000 range from 16.91℃ – 35.34o℃. Figure 5 shows that high average land surface temperature of above 30℃ was more pronounced in Ibadan North, Ibadan Northeast, Ibadan Southwest, and Ibadan Northwest, while the average land surface temperature of the fringe of the five local government areas was between 16.1℃ - 26.01℃. The difference between the minimum average land surface temperature (LST) and the maximum average land surface temperature was 18.43℃. The difference in land surface temperature in the year 2000 signifies the effect of urban heat island in the core of Ibadan because the difference in the land surface temperature of the innermost core and the periphery is above 12℃ (Senanayake et al., 2013; Voogt, 2002).

Source: Authors’ Analysis, 2023

Figure 5. Average Land Surface Temperature of the traditional core of Ibadan between 2000 and 2020

In the year 2013, an increase in the land surface temperature was recorded in the core of Ibadan. The average land surface temperature in the core of Ibadan ranges from 21.62℃ - 36.62℃ with a difference of 15℃ (Figure 5). In this epoch (2013), an average land surface of above 33.63℃ was recorded in the innermost core of four local government areas (Ibadan North, Ibadan Northeast, Ibadan Northwest and Ibadan Southwest) of the core of Ibadan. The high average land surface temperature recorded in these four local government areas was characterised by high built-up areas, urban cluster patterns, and an increase in urban population. The high average land surface temperature recorded in the innermost core also signifies the effect of urban heat island in the heart of the city.  Ibadan Southeast was the only local government area with a relatively low average land surface temperature of between 32.26℃ and 33.00 ℃, while patches of very low average land surface temperature (below 26℃) were recorded at the edge of the core of Ibadan (Figure 5).

A dynamic in the average land surface temperature in the traditional core of Ibadan was observed in the year 2020, with an average land surface temperature of 22.64℃ - 31.86℃ and a difference of 9.22℃. The deceased in the land surface temperatures in 2020 compared to other study epochs (2000 and 2013) was attributed to a decrease in anthropogenic activities due to the 2020 Corona Virus Disease (COVID-19) pandemic lockdown. Alqasemi et al. (2021) affirm this claim by opining that the 2020 pandemic was responsible for the shutdown of anthropogenic activities, which reduced other sources of emission that could lead to high LST in the urban area. Figure 5 reveals that a high average land surface temperature of 29.33℃ and above was recorded innermost core of Ibadan Northwest, Ibadan North, and Ibadan Northeast. The spatial areas of the average land surface temperature of 29.33℃ and above were more evident in Ibadan Northeast. This implies that the effect of urban heat island will be felt more in Ibadan Northeast.  An increase in urban population and an increase in built-up area density was attributed to this phenomenon. The status quo in the average land surface temperature in Ibadan Southeast in the year 2020 was still the same as that of the year 2013. An average land surface temperature of 28.57℃ – 26.22℃ was recorded for Ibadan Southeast.

This study further revealed a strong positive correction between the average maximum land surface temperature and the population density of the traditional core of Ibadan within the study epoch (see figure 6). The figure reveals that population density of the core of Ibadan is also a driver and an important indicator that has affect the average maximum land surface temperature of the study areas. These findings align Chen et al. (2016) and Li et al. (2014) assertions that population density has an effective on the average land surface temperature of a residential area. The coefficient of determinant (r2) recorded within the study period were 0.5382, 0.0044, and 0.03053 for the year 2000, 2013 and 2020 respectively.

Source: Authors’ Analysis, 2023

Figure 6. Correction between Population density and LST for the study epoch

Normalized Difference Vegetation Index (NDVI)

Several vegetation indices are available for vegetation extraction, but the most widely used vegetation index is the Normalized Difference Vegetation Index (Jensen, 2000; Taufik & Ahmad, 2016; Kaplan et al., 2018). The Normalized Difference Vegetation Index range from -1 to +1 with 0 indicating bare surfaces or areas with no vegetation. Areas with green vegetation show an NDVI value close to + 1, while non-vegetated areas show a value of – 1 (Senanayake et al., 2013). The three-epoch understudy reveals the NDVI values range from 0.4726 to 0.0450, 0.5096 to - 0.0412, and 0.3476 to 0.0149 for the years 2000, 2013, and 2020 respectively (See figure 7). The NDVI values show that as the anthropogenic activities increases in the traditional core of Ibadan, significant vegetated areas are loss. This result affirmed Folorunsho et al. (2017) assertion that NDVI values decrease with an increase in urban development.

Source: Authors’ Analysis, 2020

Figure 7. NDVI of the Core of Ibadan in year 2000, 2013 and 2020

Relationship between Land Surface Temperature, Normalized Difference Built-up Index and the Normalized Difference Vegetation Index

The correlation was used to determine the relationship between average land surface temperature and normalized difference built-up index, average land surface temperature, and normalized difference vegetation index of the study area (Table 4). The relationship between average land surface temperature and the normalized difference built-up index shows a strong positive correlation (See Figure, 8). The strong positive correlation implies that the increasing built-up areas in the traditional core of Ibadan will strengthen the adverse effects of urban heat island experienced in the heart of Ibadan core. The coefficients of determinants of the three epochs (2000, 2013, and 2020) understudy are 0.9995, 0.9534 and 0.9293, respectively.

Table 4. Relationship between LST and NDBI index, LST and NDVI index

| Year | LST versus NDBI |  | LST versus NDVI |  |
| --- | --- | --- | --- | --- |
|  | Coefficient of correlation (r) | Coefficient of determinant (r2) | Coefficient of correlation (r) | Coefficient of determinant (r2) |
| 2000 | 0.9997 | 0.9995 | - 0.9752 | - 0.9511 |
| 2013 | 0.9764 | 0.9534 | - 0.9942 | - 0.9885 |
| 2020 | 0.9640 | 0.9293 | - 0.9496 | - 0.9018 |

Source: Authors’ Analysis, 2023

Source: Authors’ Analysis, 2023

Figure 8. correction between NDVI and NDBI for the year 2000, 2013 and 2020

Elevated land surface temperature and the removal of vegetated land cover adversely affect the surrounding environment (Senanayake et al., 2013). The relationship between average land surface temperatures and the normalized difference vegetation index was a strong negative correlation in the epochs of the study (Table 5). The negative correlation between average land surface temperatures and the normalized difference vegetation index in the core of Ibadan LST reveals that vegetated land cover can weaken the effects of urban heat islands experienced in the inner core (See figure 9). This finding affirmed Fabeku et al. (2018) claims that improved vegetated land cover or greening in a geographical location will reduce the adverse effects of land surface temperature within the region.

Source: Authors’ Analysis, 2023

Figure 9. Correction between NDVI and AVG_LST for the year 2000, 2013 and 2020

Conclusion

This study has effectively analysed the distribution pattern of the land surface temperature of the traditional core of Ibadan between 2000 and 2020. The study reveals that an increase in anthropogenic activities resulting from rapid urbanisation has adversely altered the natural landscape in the traditional core of Ibadan by increasing the land surface temperature. This alteration manifests in converting vegetation land covers into physical developments and other impervious surfaces by the increasing urban population. This occurrence, in turn, contributes to the magnitude of urban heat island in Ibadan's core, as revealed in 2000 and 2013 epochs.

The study also shows that a reduction in anthropogenic activities in a region is consequential to a decrease in the land surface temperature of that region. Though the core of Ibadan had a compact formation, the 2020 pandemic (COVID-19) restriction of anthropogenic activities significantly decreased land surface temperature. The relationship between this study's average LST and NDBI further reveals that increased built-up areas will strengthen the adverse effects of urban heat islands. Future studies can be carried out on LST in the traditional core of Ibadan by modelling and simulating the urban heat island of the core. Therefore, this study recommends that tree planting, rooftop gardens, and other soft landscape elements be encouraged to minimise the effects of urban heat islands experienced in the inner core of Ibadan.

References

Abegunde, L. & Adedeji, O.  (2015). Impact of Land use Change on Surface Temperature in Ibadan, Nigeria. International Journal of Environmental and Ecological Engineering, 9(3), 1-7.

Adelekan, I. (2016). Urban Africa Risk Knowledge: Ibadan City Diagnostic Report. Working Paper No.4, 1-21.

Adetoro, O. & Salami, A. (2018). Assessment of Land Surface Temperature and LULC Changes and the Antecedent Flood Hazard in Ibadan, Nigeria. Journal of Ecology & Natural Resources, 2(2), 1-11.

Alqasemi, A. S., Hereher, M. E., Kaplan, G., Al-Quraishi, A. M. F., & Saibi, H. (2021). Impact of COVID-19 lockdown upon the air quality and surface urban heat island intensity over the United Arab Emirates. Science of the Total Environment, 767, 144330.

Arimah, B. (1994). The nature and determinants of inequalities in housing amenities in an African city. In: Albert, I. O. et al (Eds.) Urban Management and Urban Violence in Africa. (pp. 119-131) Ibadan: IFRA.

Chang, X., Wang, D., Xing, Y., Wang, J., & Gong, W. (2023). Dynamic Responses of Landscape Pattern and Vegetation Coverage to Urban Expansion and Greening: A Case Study of the Severe Cold Region, China. Forests, 14(4), 801.

Chen, L., Jiang, R., & Xiang, W. (2015). Surface Heat Island in Shanghai and Its Relationship with Urban Development from 1989 to 2013. Advances in Meteorology, 2016, 1-15

Fabeku, B., Balogun, I., Adegboyega, S., & Faleyimu, O. (2018). Spatio-Temporal Variability in Land Surface Temperature and Its Relationship with Vegetation Types over Ibadan, South-Western Nigeria. Atmospheric and Climate Sciences, 8, 318-336.

Fang, L., Wang, L., Chen, W., Sun, J., Cao, Q., Wang, S., & Wang, L. (2021). Identifying the impacts of natural and human factors on ecosystem service in the Yangtze and Yellow River Basins. Journal of Cleaner Production, 314, 127995.

Fashae, O., Adagbasa, E., Olusola, A., & Obateru, R. (2020). Land use/land cover and Land Surface Temperature of Ibadan and environs, Nigeria. Environ Monit Assess, 192(2), 109 https://doi.org/10.1007/s10661-019-8054-3

Folorunsho, A., Balogun, I., Adediji, A., Olumide, A., & Abdulkareem S. (2017). Assessment of Urban Heat Island over Ibadan Metropolis Using Landsat and Modis. International  Journal of Environment and Bioenergy, 12(1), 62-87.

Fourchard, L. (2003). Urban Slums Reports: The case of Ibadan, Nigeria. In: Understanding Slums: Case Studies for the Global Report on Human Settlements 2003.

Jensen, J. (2000) Remote Sensing of the Environment: An Earth Resource Perspective. Pearson Education, Inc., Delhi, 361-365.

Kafi, K., Shafri, H., & Shariff, A. (2014). An Analysis of LULC Change Detection Using Remotely Sensed Data; A Case Study of Bauchi City. IOP Conference Series: Earth and Environmental Science, 20, 12-56.

Kaplan, G., Avdan, G., & Avdan, Z. (2018). Urban Heat Island Analysis Using the Landsat 8 Satellite Data: A Case Study in Skopje, Macedonia. Proceedings, 358(2), 1-5.

Lin, L., Yongbin, T., Shen, Y., Zhonghai Y., Zhen, L., & Honghao, L. (2014). Impact of Land Cover and Population Density on Land Surface Temperature: Case Study in Wuhan, China. Journal of Applied Remote Sensing, 8, 1-19.

NASA. (2012). Landsat 7 Science Data Users Handbook. NASA.

Nuruzzaman, M. (2015). Urban Heat Island: Causes, Effects and Mitigation Measures - A Review. International Journal of Environmental Monitoring and Analysis, 3(2), 67-73.

Nwosu, N., & Oshunsanya, S. O. (2020). Management of Agricultural Lands for Sustainable Crop Cultivation and Greenhouse Gas Reduction. Handbook of Climate Change Management: Research, Leadership, Transformation, 1-28.

Olorunfemi, I., Fasinmirin, J., Olufayo, A., & Komolafe, A. (2020). GIS and remote sensing-based analysis of the impacts of land use/land cover change (LULCC) on the environmental sustainability of Ekiti State, southwestern Nigeria. Environment, development and sustainability, 22, 661-692.

Ogunbode, T. & Ifabiyi, I. (2019) Relationship between Evapotranspiration and water Availability in the Tropical Regions: A Case Study of a South Western City in Nigeria. Archives of Current Research International, 16(4), 1-10.

Onibokun, A., & Kumuyi, A. (1999). Ibadan, Nigeria. In A.G. Onibokun (Ed.), Managing the Monster: Urban Waste and Governance in Africa (pp 49–100). International Development Research Centre, Ottawa.

Popoola, A. (2019). Climate Variability and Urban Agricultural Activities in Ibadan, Nigeria. In A.K., Rathoure & P.B., Chauhan (Eds.), Current State and Future Impacts of Climate Change on Biodiversity (pp.11-30). IGI Global: USA.

Senanayake, I., Welivitiya, W., & Nadeeka, P. (2013). Remote sensing based analysis of urban heat islands with vegetation cover in Colombo city, Sri Lanka using Landsat-7 ETM+ data. Urban Climate, 5, 19–35.

Taufik, A. &Ahmad, S. (2016). Land covers classification of Landsat 8 satellite data based on Fuzzy Logic approach IOP Conf. Ser.: Earth Environ. Sci. 37, 1-7.

United Nations Human Settlements Programme (UN-Habitat), (2009). Planning Sustainable Cities: Global Report on Human Settlements. Earthscan publishes in association with the International Institute for Environment and Development.

Voogt, J. (2002). Urban Heat Island. In Munn, T. (ed.) Encyclopedia of Global Environmental Change, Vol. 3. Chichester: John Wiley and Sons.

Wahab, B. & Popoola, A. (2019). Urban Farmers’ Perceptions of and Adaptation Strategies to Climate Variability in Ibadan, Nigeria. In P., Cobbinah & M., Addaney (Eds.), The Geography of Climate Change Adaptation in Urban Africa (pp. 123-154). Palgrave Macmillan, Cham: Switzerland. https://doi.org/10.1007/978-3-030-04873-0_5

Weng, Q., Lub, D., & Schubring, J. (2004). Estimation of Land Surface Temperature Vegetation Abundance Relationship for Urban Heat Island Studies. Remote Sensing of Environment, 89, 467-483.

Zahabi, M., & Pradhan, B. (2017). Effect of Urban Expansion on Land Surface Temperature in Putrajaya City, Malaysia. In B. Pradhan (Ed.), Spatial Modeling and Assessment of Urban Form, Analysis of Urban Growth: From Sprawl to Compact Using Geospatial Data (pp. 61-92). Springer.

Zhang, D., Shou, Y., Dickerson, R., & Chen, F. (2011). Impact of Upstream Urbanization on the Urban Heat Island Effects along the Washington–Baltimore Corridor. Journal of Applied Meteorology and Climatology, 50, 2012-2029.